# Segments

- Anchor Bolts segment

input

- Nb = total number of anchor bolts

- db = nominal anchor bolt diameter (in)

- dh = anchor bolt hole diameter (in)

- rb = anchor bolt circle radius (ft or in)

- αb = start angle of first anchor bolt (deg)

- θseg = segment angle (deg), if bolts are checked by segment

- ns = number of segments

- tbp = base plate thickness (in)

- fy = bolt yield strength, if needed

- fu = bolt ultimate strength, if needed

- Tu = design uplift/tension demand on one bolt or bolt group

- Vu = design shear demand on one bolt or bolt group

- φ = resistance factor, if your software includes strength check

- e = edge distance from hole to plate edge, if required

- s = bolt spacing, if user enters directly

- nbs = number of bolts per segment

output

- Bolt cross-sectional area

If we consider nominal diameter

Ab= πdb24 inch2

2) Hole area

Ah= πdh24

3) Bolt angular spacing around full circle

Δθb= 360Nb

4) Angle of each bolt

For bolt i:

θi= αb+i-1∆θb

5) Coordinates of each anchor bolt

If on bolt circle radius rb:

xi= rbcosθi

yi= rbsinθi

6) Arc spacing between adjacent bolts

Sarc=rb∆θb π180

7) Chord spacing between adjacent bolts

This is usually more practical than arc spacing:

Schord=2rbsin∆θb2

8) Number of bolts per segment

nbs= Nbns

This should be an integer.

If not, the software should flag a warning.

9) Bolt hole clear edge distance check

If the hole is located between plate edges, the minimum clear edge distance can be written as:

eclear=e- dh2

10) Tension demand per bolt

If total uplift is shared equally:

Tbolt= TuNb

Tbolt= TuNeffi

If only some bolts resist uplift we are using the last equation where Neffi=effictive number of bolts in tension.

11) Shear demand per bolt

If total shear is shared equally:

Vbolt= VuNb

12) Tensile capacity of one bolt

Basic form:

Pnt=AbFnt

or design strength:

∅Pnt=∅AbFnt

If we use ultimate strength:

∅Pnt=∅AbFu

13) Shear capacity of one bolt

Vn=AbFnv

or design strength:

∅Vn=∅AbFnv

14) Interaction check for combined tension and shear

A simple software form can be:

Utilization= Tbolt∅Pnt + Vbolt∅Vn

Pass if:

Utilization≤1.0

- Base plate segment

input

- Dbp = base plate diameter (ft or in)

- Ro = base plate outside radius

- Ri = base plate inside radius

- θ = base plate segment angle (deg)

- t = base plate thickness (in)

- n = number of base plate segments

- rs = steel unit weight = 490 pcf

- Nh = number of anchor bolt holes in one segment

- dh = diameter of anchor bolt hole

- α = start angle of segment (deg), if you want location in plan

- rb = bolt circle radius, if bolt layout is needed

Output

- Gross area, Ag

Ag= θ360π R02- Ri2

- Net area, An

An=Ag- Nh πdh24 1144

- Volume, V

V= Agt12

- Weight per segment, Wbp

Wbp= Ag( t12) ( rs1000

- Total weight, Wbp,total

Wbp,total=n .Wbp

- Outer arc length, Lo

Lo =R0 .θr

- Inner arc length, Li

Li =Ri .θr

θr= θπ180

- Radial width, b

b =R0-Ri

- Centroid radius, r

For one segment, the centroid lies on the segment centerline:

r = 4sinθr​23θs x R03- Ri3R02- Ri2

r= radial location of centroid from center

θs= segment angle (in radians)

- Centroid angle of segment

β= ∝ + θ2

β = centroid angle (location of centroid in angular direction)

α = starting angle of the segment

θ = segment angle

Centroid coordinates, xcyc

xc= rcosβ

yc= rsinβ