# Segments

Base plate segment

input

- Dbp = base plate diameter (ft or in)

- Ro = base plate outside radius

- Ri = base plate inside radius

- θ = base plate segment angle (deg)

- t = base plate thickness (in)

- n = number of base plate segments

- rs = steel unit weight = 490 pcf

- Nh = number of anchor bolt holes in one segment

- dh = diameter of anchor bolt hole

- α = start angle of segment (deg), if you want location in plan

- rb = bolt circle radius, if bolt layout is needed

Output

- Gross area, Ag

Ag= θ360π (R02- Ri2)

- Net area, An

An=Ag- Nh( πdh24 )1144

- Volume, V

V= Ag(t12)

- Weight per segment, Wbp

Wbp= Ag( t12) ( rs1000

- Total weight, Wbp,total

Wbp,total=n .Wbp

- Outer arc length, Lo

Lo =R0 .θr

- Inner arc length, Li

Li =Ri .θr

θr= θ(π180)

- Radial width, b

b =R0 .Ri

- Centroid radius, r

For one segment, the centroid lies on the segment centerline:

r = 4sin(θr​/2)3θs x R03- Ri3R02- Ri2

- Centroid angle of segment

β= ∝ + θ2

- Centroid coordinates, xcyc

xc= rcosβ

yc= rsinβ