# Segments

- Anchor Bolts segment

input

- Nb = total number of anchor bolts

- db = nominal anchor bolt diameter (in)

- dh = anchor bolt hole diameter (in)

- rb = anchor bolt circle radius (ft or in)

- αb = start angle of first anchor bolt (deg)

- θseg = segment angle (deg), if bolts are checked by segment

- ns = number of segments

- tbp = base plate thickness (in)

- fy = bolt yield strength, if needed

- fu = bolt ultimate strength, if needed

- Tu = design uplift/tension demand on one bolt or bolt group

- Vu = design shear demand on one bolt or bolt group

- φ = resistance factor, if your software includes strength check

- e = edge distance from hole to plate edge, if required

- s = bolt spacing, if user enters directly

- nbs = number of bolts per segment

output

- Bolt cross-sectional area

If we consider nominal diameter

Ab= πdb24 inch2

2) Hole area

Ah= πdh24

3) Bolt angular spacing around full circle

Δθb= 360Nb

4) Angle of each bolt

For bolt i:

θi= αb+i-1∆θb

5) Coordinates of each anchor bolt

If on bolt circle radius rb:

xi= rbcosθi

yi= rbsinθi

6) Arc spacing between adjacent bolts

Sarc=rb∆θb π180

7) Chord spacing between adjacent bolts

This is usually more practical than arc spacing:

Schord=2rbsin∆θb2

8) Number of bolts per segment

nbs= Nbns

This should be an integer.

If not, the software should flag a warning.

9) Bolt hole clear edge distance check

If the hole is located between plate edges, the minimum clear edge distance can be written as:

eclear=e- dh2

10) Tension demand per bolt

If total uplift is shared equally:

Tbolt= TuNb

Tbolt= TuNeffi

If only some bolts resist uplift we are using the last equation where Neffi=effictive number of bolts in tension.

11) Shear demand per bolt

If total shear is shared equally:

Vbolt= VuNb

12) Tensile capacity of one bolt Basic form:

Pnt=AbFnt

or design strength:

∅Pnt=∅AbFnt

If we use ultimate strength:

∅Pnt=∅AbFu

13) Shear capacity of one bolt

Vn=AbFnv

or design strength:

∅Vn=∅AbFnv

14) Interaction check for combined tension and shear

A simple software form can be:

Utilization= Tbolt∅Pnt + Vbolt∅Vn

Pass if:

Utilization≤1.0

- Base plate segment

input

- Dbp = base plate diameter (ft or in)

- Ro = base plate outside radius

- Ri = base plate inside radius

- θ = base plate segment angle (deg)

- t = base plate thickness (in)

- n = number of base plate segments

- rs = steel unit weight = 490 pcf

- Nh = number of anchor bolt holes in one segment

- dh = diameter of anchor bolt hole

- α = start angle of segment (deg), if you want location in plan

- rb = bolt circle radius, if bolt layout is needed

Output

- Gross area, Ag

Ag= θ360π R02- Ri2

- Net area, An

An=Ag- Nh πdh24 1144

- Volume, V

V= Agt12

- Weight per segment, Wbp

Wbp= Ag( t12) ( rs1000

- Total weight, Wbp,total

Wbp,total=n .Wbp

- Outer arc length, Lo

Lo =R0 .θr

- Inner arc length, Li

Li =Ri .θr

θr= θπ180

- Radial width, b

b =R0-Ri

- Centroid radius, r

For one segment, the centroid lies on the segment centerline:

r = 4sinθr​23θs x R03- Ri3R02- Ri2

r= radial location of centroid from center

θs= segment angle (in radians)

- Centroid angle of segment

β= ∝ + θ2

β = centroid angle (location of centroid in angular direction)

α = starting angle of the segment

θ = segment angle

Centroid coordinates, xcyc

xc= rcosβ

yc= rsinβ

3. Ring Wall Foundation Segment (Main Structural Element)

1. Input Data

A. Geometry Input

- trw = ring wall thickness (ft)

- B = footing base width (ft)

- Rcl = radius to foundation centerline (ft)

- TCE = top of concrete elevation (ft)

- tedge = footing edge thickness (ft)

- cc = concrete cover to reinforcement (in)

- Rin = inner radius

- Rout = outer radius

- A = footing plan area

- Rrw = ring wall centerline radius

B. Material Input

- f'c = concrete compressive strength

- fy = reinforcement yield strength

- γc = concrete unit weight

- λ = lightweight concrete factor, if applicable

C. Geotechnical Input

- qallow = allowable soil bearing pressure

- μ = friction coefficient, if needed

D. Loading Input

- Vwater = water volume

- γw = unit weight of water

- Wtank = tank dead load

- Wsup = superimposed load

- Mu = factored flexural moment for footing strip

- Vu = one-way shear demand

- Vup = punching shear demand

- Bu = concrete bearing demand

- A1 = loaded area for bearing

- A2 = supporting area for bearing

- b0 = critical punching perimeter

3.1 Geometry Calculation

1) Inner radius of footing

Rin=Rcl-B2

2) Outer radius of footing

Rout=Rcl+B2

3) Footing plan area

A=πRout2Rin2

4) Ring wall centerline radius

Rrw=Rcl+trw2
3.2 Service Load Calculation

5) Water weight

Ww=Vwater7.48052⋅γw1000

6) Tank dead load

Wtank=input value

7) Superimposed load

Wsup=input value

8) Footing self-weight

Wf=A⋅tedge⋅γc1000

9) Total service vertical load

Vs=Ww+Wtank+Wsup+Wf

10) Allowable soil bearing pressure

qallow=input allowable soil bearing1000

11) Service bearing pressure

qs=VsA

12) Bearing utilization ratio

Ub=qsqallow

Bearing requirement

Ub≤1.0

3.3 Flexural Design of Ring Footing Strip

This part is based on a 1-foot circumferential strip.

13) Effective depth

d=tedge×12-cc

14) Strip width

b=12 in

15) Concrete strength

fc'=input value

16) Steel yield strength

fy=input value

17) Lightweight factor

λ=input value

18) Beta one factor

β1=0.85if fc'≤4000max0.65,  0.85-0.05fc'-40001000if fc'>4000

19) Yield strain

εty=fyEs

20) Compression block depth

Use the same form implemented in the Excel file:

a=Asfy0.85 b

21) Neutral axis depth

c=aβ1

22) Net tensile strain

εt=0.003d-cc

23) Strength reduction factor

ϕ=0.90if εt≥εty+0.0030.65if εt≤εty0.65+0.25εt-εty0.003otherwise

24) Nominal moment capacity

Mn=Asfyda2

25) Design moment strength

ϕMn=ϕMn/1000

26) Flexural utilization ratio

Uf=MuϕMn

Flexural requirement

Uf≤1.0

27) Required nominal moment for steel design

Mn,req=Mu×10000.9

28) Required steel area

As,req=fyd-fyd)24fy22(0.85fc'b)Mn,req2fy22(0.85fc'b)

29) Provided steel area

As,prov=Abar×12sprov

30) Required bar spacing

sreq=Abar×12As,req

3.4 One-Way Shear Check

31) Effective depth

d=from flexure section

32) Strip width

b=12 in

33) Total member thickness

h=tedge×12

34) Size effect factor

λs=min⁡121+d/10

35) Reinforcement ratio

ρw=Asbd

36) Gross concrete area

Ag=bh

37) Nominal one-way shear strength

Use the exact expression from the Excel logic.

38) Design one-way shear strength

ϕVn=ϕVn/1000

39) One-way shear utilization ratio

Uv=VuϕVn

One-way shear requirement

Uv≤1.0

3.5 Punching Shear Check

40) Critical perimeter

b0=input value
41) Beta factor

β=1

42) Column location factor

αs=40Interior30Edge20Corner
43) Punching shear stress limits

Use the three expressions from the Excel logic and take the minimum controlling value:

vc1=4λfc'
vc2=24βλfc'
vc3=2αsdb0λfc'
44) Governing punching shear stress

vc=min⁡(vc1,vc2,vc3)

45) Nominal punching shear strength

Vn,p=vc b0 d

46) Design punching shear strength

ϕVn,p=ϕVn,p/1000

47) Punching shear utilization ratio

Up=VupϕVn,p

Punching requirement

Up≤1.0

3.6 Concrete Bearing Check

48) Nominal concrete bearing strength

If bearing enhancement is permitted:

Rn=min⁡0.85fc'A1A2A1,  2(0.85fc'A1)

If enhancement is not permitted:

Rn=0.85fc'A1

49) Design concrete bearing strength

ϕRn=ϕRn/1000

50) Concrete bearing utilization ratio

Ubr=BuϕRn
Bearing requirement

Ubr≤1.0