# Segments

Cylindrical segment

input

D = diameter (in)

hi= initial height (ft)

hf = final height (ft)

H = total height (ft)

t = thickness (in)

output

Ap = projected area (ft2) = 12(h)(d)

Centroid of projected area = hi + h/2

Weight of the pedestal =  π4 (D2 – (D-2*t/12)2) H rs/1000 (kips)

kzi = 2.01(hi /Zg)2/a

kzf= 2.01(hf /Zg)2/a

qzi = 0.00256 Kzi Kzt Kd I V2                 , other parameters shall be obtained from wind load

qzf = 0.00256 Kzf Kzt Kd I V2

F = qzi+qzt2 AP

L= lever arm between the base and the location of the force F= hi + qzih2/2+0.5(qzf-qzi)2h2/3qzih+0.5qzf-qzih

M base = L * F

Conical segment

input

Di = initial diameter (ft)

Df = Final diameter (ft)

hi= initial height (ft)

hf = final height (ft)

H = total height (ft)

t = thickness (in)

output

Ap = projected area (ft2) = (h)(d)

Centroid of projected area = hi + h/2

S1 = π4Di2

S2 = π4Df2

S3 = π4(Di-t/12)2

S4 = π4(Df-t/12) 2

W = (H/3 *( S1 + S2 - S3 - S4 +S1S2 -S3S4)) rs/1000 (kips)

kzi = 2.01(hi /Zg)2/a

kzf= 2.01(hf /Zg)2/a

qzi = 0.00256 Kzi Kzt Kd I V2                 , other parameters shall be obtained from wind load

qzf = 0.00256 Kzf Kzt Kd I V2

F = qzi+qzf2 AP

L= lever arm between the base and the location of the force F= hi + qzih2/2+0.5(qzf-qzi)2h2/3qzih+0.5qzf-qzih

M base = L * F

# Snow Load

- Ground snow load Pg shall be based on ASCE 7

- Flat roof snow load Pf = 0.7 Ce Ct Is Pg

- Ce = 0.9 for wind Exposure C

- = 0.8 for wind Exposure D

- Ct = 1.2 for unheated and open-air structures

- Is  = 1.0 for Risk Category II

- = 1.1 for Risk Category III

- = 1.2 for Risk Category IV

- Slopped roof snow load

- Wherever the angle between the tangent of the tank and the horizontal axis is greater than 70o the snow load shall be zero. (They assume it on half the diameter.)

- Cs = (70- a)/55

- Ps = Cs Pf

- The minimum snow load is 0.96 N/m2 (20 psf)

# balconies, platforms, ladders, stairs

- 1,000 lb (454 kg) to any 10 ft² (0.93 m²) area on balcony floors.

- 1,000 lb (454 kg) to each platform.

- 500 lb (227 kg) to any 10 ft² (0.93 m²) area on tank roofs.

- These loads need not be combined with design snow load requirements.

- These loads need not be applied simultaneously.

# Content weight

- Wcontent = V rw

# Constants

- rw = 62.4 pcf

- rs = 490 pcf

- rc = 144 pcf

# Lateral Loads

# Wind load

- qz = Y lw 0.000256 Kz Kzt Kd I V2

- lw = strength-level to service-level factor for wind load

- Kd = directionality factor 1.0

- Y=0.6

- V = basic wind speed

- Kzt = Topography factor normally (1.0)

- Kz = Velocity pressure factor. 2.01(Z/Zg)2/a

- Iw = 1.15 (We are not going to use this I)

- Pw = qz G Cf  ≥ 30 Cf

- F = Pw Af

- Af = p a b

# Seismic load

- SDS = 2/3 SMS

- SD1 = 2/3 SM1

- SMS = Fa Ss

- SM1 = Fv S1

- S1 & S1 are the mapped spectral acceleration parameters

## Design response spectrum for impulsive component

Ti ≤ Ts                                    Sai = SDS

Ts ≤ Ti ≤ TL                        Sai = SD1 / Ti  ≥ SDS

Ti ≥ TL                                   Sai = SD1 TL / Ti2

Ts = SD1/SDs

- Ai = λESaiIERi ≥ 0.36 S1IERi

- λE is the strength-level to service-level factor for seismic = 0.70

- V = Ai W

- W is the weight of the structure plus the content.

- Force V applies at the tank's center of mass.

## Critical questions

- How do you calculate the weight of water and structure? (This question is just to be on the same page.)

- How to accurately find Cf?

- How to calculate the fundamental period of the structure? Codes and standards recommend using the short period for the impulsive component. However, in the calculation books, they were putting a value for the period without telling how.